# Report

# 1. UNSW Lower Campus

## 1.1 Datasets

| Data | Size | #Voxels | Description |
| --- | --- | --- | --- |
| dtmbot.xyz | 9.22 GB | 641,624,355 | A terrain with holes, in which the buildings fit |
| tree.xyz | 906 MB | 59,640,000 | Tree in lower campus |
| bld1-54.xyz (except for 26 and 46) | 3.52GB | 241,613,693 in total and 4,646,418 per building | 52 buildings in lower campus |
| be.xyz | 249 MB | 17,460,029 | Built Environment (H13) |
| blockhouse.xyz | 45.4 MB | 3,392,202 | Blockhouse (G6) |
| dalton.xyz | 25.5 MB | 1,887,512 | Dalton (F12) |
| quadrangle.xyz | 43.9MB | 3,161,733 | Quadrangle (E15) |
| roundhouse.xyz | 79.9MB | 6,037,174 | Roundhouse (E6) |
| scithe.xyz | 17.2MB | 1,231,821 | Science Theatre (F13) |

Note that:

- For GIS-based voxels, its resolution is 20cm. All voxels are recorded in same relative coordinate with offset (336000, 6245250, 20).

- For BIM-based voxels, its resolution is 10 cm. Each building is in its own relative coordinate with MINXYZ.

  - For be.xyz, the offset is  (336300, 6245507, 25).

  - For blockhouse.xyz, the offset is (336042, 6245613, 27).

  - For dalton.xyz, the offset is (336305, 6245569, 29).

  - For quadrangle.xyz, the offset is (336409, 6245580, 31).

  - For roundhouse.xyz, the offset is (336047, 6245651, 25).

  - For scithe.xyz, the offset is (336325, 6245582, 28).

## 1.2 Database Schema

Figure 1. Conceptual Model

Figure 2. Physical Model

## 1.3 Data Layout in PostgreSQL

Four Main Tables for voxel storage:

| Table | Columns | Geometry | Description |
| --- | --- | --- | --- |
| voxel | id, x, y, z, classid, ifcid | N/A | One voxel per row/record. |
| voxelpt | id, classid, ifcid, geom | POINT(x,y,z) | One voxel per row/record with geometry. |
| voxelmpt | id, classid, geom | MULTIPOINT | One building per row/record. 20,000,000 point per row/record for “tree” and “dtm”. |
| voxelpatch | id, classid, pa | PCPATCH(1) | One building per row/record. 20,000,000 point per row/record for “tree” and “dtm”. |

Two Semantic Tables for IFC and class info:

| Table | Columns | Description |
| --- | --- | --- |
| ifcclass | ifcid, name | 26 IFC class with corresponding name |
| objclass | classid, name | classid 1-54 (except for 26 and 46) are building ID. classid 55 is tree ID. classid 56 is dtmbot ID. |

# 2. QGIS Visualization

The data query is processed using a HP laptop. Its processor is Intel(R) Core (TM) i7-7600 CPU @ 2.80GHz and its installed memory is 16.0 GB. Its operating system is 64-bit Windows 10. And the test is performed on PostgreSQL (11.2), PostGIS (2.5.2), and QGIS (3.6.1).

## 2.1 Sample Visualization

Considering bld52 (we don’t have building name at this moment), 29.6MB and 1,881,847 voxels, we extract “geom” first, and then convert its coordinate into EPSG:28356.

- SELECT ST_MakePoint(336000+ST_X(geom)*0.2, 6245250+ST_Y(geom)*0.2, 20+ST_Z(geom)*0.2) AS geom

- FROM voxelpt

- WHERE classid=52;

Figure 3 shows how to execute query in QGIS.

Figure 3. Query execution in QGIS

Note that, loading the above query result as a new layer in QGIS may take several minutes and 3D view is as well. Once choosing 3D view, please keep an eye out for your GPU and memory changes, if you crash, kill the task or stop doing the work at hand and continue to wait patiently. If not necessary or you are not confident in your PC, don’t try 3D view.

Figure 3. 2D and 3D visualization of  “bld52” in QGIS

# 3. Object Matching between Different Data Source

## 3.1 Checking Data Info

| Data | #Voxels |
| --- | --- |
| bld1-54 | 241,613,693 in total and 4,646,418 per building |
| tree | 59,640,000 |
| dtmbot | 641,624,355 |
| Ifcid is not null | 33,170,471 |
| be | 17,460,029 |
| blockhouse | 3,392,202 |
| dalton | 1,887,512 |
| quadrangle | 3,161,733 |
| roundhouse | 6,037,174 |
| scithe | 1,231,821 |

## 3.2 Assign Temporary classID for IFC buildings

In table “voxel” and “voxelpt”, GIS data occupied 944,110,209 rows.

At this moment, we assume the 6 IFC models with classID 57, 58, 59, 60, 61, and 62, respectively.

- \COPY voxel(x, y, z, ifcID) FROM 'C:\Users\z5039792\Documents\Vox3DMod\data\bim\BE\classmodel.xyz' DELIMITER ' ';

- UPDATE voxel SET classID=57 WHERE classid IS NULL;

- \COPY voxel(x, y, z, ifcID) FROM 'C:\Users\z5039792\Documents\Vox3DMod\data\bim\BlockHouse\classmodel.xyz' DELIMITER ' ';

- UPDATE voxel SET classID=58 WHERE classid IS NULL;

- \COPY voxel(x, y, z, ifcID) FROM 'C:\Users\z5039792\Documents\Vox3DMod\data\bim\Dalton\classmodel.xyz' DELIMITER ' ';

- UPDATE voxel SET classID=59 WHERE classid IS NULL;

- \COPY voxel(x, y, z, ifcID) FROM 'C:\Users\z5039792\Documents\Vox3DMod\data\bim\Quadrangle\classmodel.xyz' DELIMITER ' ';

- UPDATE voxel SET classID=60 WHERE classid IS NULL;

- \COPY voxel(x, y, z, ifcID) FROM 'C:\Users\z5039792\Documents\Vox3DMod\data\bim\Roundhouse\classmodel.xyz' DELIMITER ' ';

- UPDATE voxel SET classID=61 WHERE classid IS NULL;

- \COPY voxel(x, y, z, ifcID) FROM 'C:\Users\z5039792\Documents\Vox3DMod\data\bim\SciThe\classmodel.xyz' DELIMITER ' ';

- UPDATE voxel SET classID=62 WHERE classid IS NULL;

Figure 4. Log info for IFC data importing

## 3.3 Update classID for IFC buildings

For BE building, through computing bld19 and its the MAX & MIN (x,y) range in EPSG:28356 CRS, it is easy to find that they are in high probability the same building.

- SELECT MAX(x)*0.1+336300 AS maxx, MIN(x)*0.1+336300 AS minx, MAX(y)*0.1+6245507 AS maxy, MIN(y)*0.1+6245507 AS miny

- FROM voxel

- WHERE classid=57;

- SELECT MAX(x)*0.2+336000 AS maxX, MIN(x)*0.2+336000 AS minX, MAX(y)*0.2+6245250 AS maxY, MIN(y)*0.2+6245250 AS minY

- FROM voxel

- WHERE classID=19;

Figure 5. (x,y) range for BE in EPSG:28356 CRS

Then, visualizing above two buildings in CloudCompare, it looks similar, at least in shape.

Figure 6. BE building in CloudCompare

Figure 7. bld19 in CloudCompare

For Blockhouse, Dalton, Quadrangle, Roundhouse, SciThe buildings, calculating (x,y) range for all buildings with classID<=54. And then retrieve same range for above 5 buildings to do matching.

- SELECT MAX(x)*0.1+336042 AS maxx, MIN(x)*0.1+336042 AS minx, MAX(y)*0.1+6245613 AS maxy, MIN(y)*0.1+6245613 AS miny

- FROM voxel

- WHERE classid=58;

- SELECT MAX(x)*0.1+336305 AS maxx, MIN(x)*0.1+336305 AS minx, MAX(y)*0.1+6245569 AS maxy, MIN(y)*0.1+6245569 AS miny

- FROM voxel

- WHERE classid=59;

- SELECT MAX(x)*0.1+336409 AS maxx, MIN(x)*0.1+336409 AS minx, MAX(y)*0.1+6245580 AS maxy, MIN(y)*0.1+6245580 AS miny

- FROM voxel

- WHERE classid=60;

- SELECT MAX(x)*0.1+336047 AS maxx, MIN(x)*0.1+336047 AS minx, MAX(y)*0.1+6245651 AS maxy, MIN(y)*0.1+6245651 AS miny

- FROM voxel

- WHERE classid=61;

- SELECT MAX(x)*0.1+336325 AS maxx, MIN(x)*0.1+336325 AS minx, MAX(y)*0.1+6245582 AS maxy, MIN(y)*0.1+6245582 AS miny

- FROM voxel

- WHERE classid=62;

- SELECT MAX(x)*0.2+336000 AS maxX, MIN(x)*0.2+336000 AS minX, MAX(y)*0.2+6245250 AS maxY, MIN(y)*0.2+6245250 AS minY

- FROM voxel

- WHERE classID<=54

- GROUP BY classID;

| Building Name | Range in (x,y) EPSG:28356 CRS |
| --- | --- |
| Blockhouse |  |
| Dalton |  |
| Quadrangle |  |
| Roundhouse |  |
| SciThe |  |

Figure 8. (x,y) range for all 52 building

Figure 9. Blockhouse

Figure 10. Dalton

Figure 11. Quadrangle

Figure 12. Roundhouse

Figure 13. Science Theatre

In summary, the corresponding bld are list in below table:

| Id | Name | Num | Old classID | New classID |
| --- | --- | --- | --- | --- |
| Bld19 | Built Environment | H13 | 57 | 19 |
| Bld6 | Blockhouse | G6 | 58 | 6 |
| Bld9 | Dalton | F12 | 59 | 9 |
| Bld13 | Quadrangle | E15 | 60 | 13 |
| Bld44 | Roundhouse | E6 | 61 | 44 |
| Bld8 | Science Theatre | F13 | 62 | 8 |

Update the 6 buildings:

- UPDATE voxel SET classID=19 WHERE classid=57;

- UPDATE voxel SET classID=6 WHERE classid=58;

- UPDATE voxel SET classID=9 WHERE classid=59;

- UPDATE voxel SET classID=13 WHERE classid=60;

- UPDATE voxel SET classID=44 WHERE classid=61;

- UPDATE voxel SET classID=8 WHERE classid=62;

Figure 14. Log info for IFC classID updating

## 3.4 Assign Name for Each Building in Lower Campus

Ignore …

# 4. Conversion to Point

In this section, we consider the second data layout, that stores each voxel as a geometry POINT including (x,y,z).

- DELETE FROM voxelpt;

- INSERT INTO voxelpt(classID, ifcID, geom) SELECT classID, ifcID, ST_MakePoint(x,y,z) FROM voxel AS VALUES;

Figure 15. Log info for voxelpt inserting

# 5. Conversion to Multipoint

## 5.1 Create table

DROP TABLE IF EXISTS voxelmpt CASCADE;

CREATE TABLE voxelmpt

(

id serial PRIMARY KEY,

classID INTEGER,

ifcID INTEGER,

geom geometry

);

## 5.2 Partition size

For each IFC object, we regard each IFC object as one Multipoint.

For each GIS building, each building is one Multipoint.

For tree and dtmbot, we combine GIS dataset to decide the patch size.

## 5.3 Data Import

# 6. Conversion to PCPATCH